|  | Pheno Seal |  |  |
| --- | --- | --- | --- |
| STOPS LOST CIRCULATION IN DRILLING |  |  |  |
| GENERAL DESCRIPTION Pheno Seal is a chemically stable thermoset laminate used to correct lost circulation in all water, oil, and synthetic-based fluids and cements. Pheno Seale is ground from laminate sheets into flakes of various sizes, which allow for maximum sealability at a very efficient dosage per barrel. APPLICATIONS Pheno Seal is used in all water, oil and synthetic drilling fluids as a premium-performance Lost Circulation Material (LCM). Pheno Seal maintains the best cost-to-benefit ratio of any LCM product, greatly outperforming nutshells, fibers, mica and other blended materials for sealing fractures. Pheno Seal can be used in any fluid-based drilling system, anywhere that significant fluid loss is a major concern. INSTALLATION Pheno Seal should be added at a rate of 10 to 30 ppb to a 25 to 50 barrel pill, mixed, and pumped downhole. Pheno Seal Fine and/or Medium gradations may be used, depending on the degree of lost circulation. Pheno Seal Coarse may be needed in severe cases of lost returns. After regaining circulation, 40 to 160 lbs. per hour (1 to 4 sacks) of Pheno Seal should be mixed to reduce fracturing of a newly drilled formation. To minimize seepage loss, 80 to 320 lbs. per hour (2 to 8 sacks) of Pheno Seal Fine should be mixed into the system while drilling. PHYSICAL PROPERTIES. Material:; Ground thermoset laminate; Color :; Varied. Form:; Hard, flat chip; Chemical Resistance:; Inert. Specific Gravity:; 1.3; Temperature range; Stable up to 500' F. Length:. PACKAGING Bags 40 lbs. (18.15 kg.) Pallets 50 bags; 2000 lbs. (909 kg.) WARRANTY Pheno Seal is warranted to be free of defects and to meet all quality control standards set by the manufacturer. SONTEK has no control over the placement or use of the fluids in which Pheno Seal is incorporated, therefore SONTEK assumes no responsibility for the end product. |

Pheno Seal

Please take a few minutes to get acquainted with Pheno Seal and our other drilling products.

GENERAL INFORMATION AND ADVANTAGES

Pheno Seal is a chemically stable thermoset laminate used for lost circulation, bit balling, and pipe and casing sliding in all water, oil, and synthetic based fluids and cements. The original laminate sheets are ground into flakes and sized to provide maximum sealability using minimum pounds per barrel. Pheno Seal is safe and effective in all water, oil, and synthetic-based muds, and is also compatible with squeezes and cements. Pheno Seal is one of the most effective loss circulation materials on the market, offering tremendous performance advantages in cases of fractured formations, seepage loss, and bit balling.

| Fractured Formations Each hard chip shaped Pheno Seal particle hydraulically rotates with the fluid and flows into fractures along the wellbore. As the particles wedge together inside the fracture, the differential pressure wedges the particles tighter together, which in turn increases the pressure resistance of the seal. Pheno Seal, in high concentrations, is an effective squeeze material. Pheno Seal's optimum particle shape and sizing provides quicker and more complete seals than blended materials. |  |
| --- | --- |
|  | Pheno Seal Sealing Fractures |
| Seepage Loss, Pipe Sliding and Running Casing Pheno Seal provides a hard, smooth, pressure resistant barrier between the drillstring and wellbore. This reduces the pressure differential along the wellbore, as well as reducing the adhesion between the drillpipe and filtercake. Pheno Seal and calcium carbonate is preferred. Unlike granular materials, Pheno Seal does not degrade filtercake quality. |  |

Bit Balling

The strong, thin edges of each Pheno Seal particle cut away at formation build-up on bits and stabilizers like a knife blade. Pheno Seal also contains approximately 7 times more particles per pound than nut plug. Pheno Seal's inherent hardness, multiple cutting edges, and optimum hydraulic shape allow for an effective and economical solution for controlling formation buildup on bits and stabilizers.

MWD, Down Hole Tools, and Mud Motors

The maximum particle size of Pheno Seal Fine is smaller than nut hulls medium. Pheno Seal Fine can be used in tools and mud motors that have approved the use of medium nut hulls. Any screens used in the drill pipe should be kept at the surface, and not placed down hole. Typical Pheno Seal particle sizing is listed in the chart below, along with a recommended minimum jet size for the the various Pheno Seal gradations.

| Typical Pheno Seal® Particle Sizing -- (% Retained) |  |  |  |  |
| --- | --- | --- | --- | --- |
| MESH | MICRON | FINE | MEDIUM | COARSE |
| -80 +80 +60 +20 +14 +10 +4 | 177 177 250 850 1190 2000 4750 | 1.6 4.0 44.9 20.6 28.9 0.0 0.0 | 0.0 0.0 0.1 0.2 25.4 74.3 0.0 | 0.0 0.0 0.0 0.2 12.4 82.2 5.2 |
| Minimum Jet Size None 9/32 16/32 |  |  |  |  |

| Pheno Seal Advantages - Best cost to benefit ratio of any LCM product. - Outperforms nutshells, fibers, mica, and blended materials for sealing fractures. - 7 times more particles per pound than granular LCM products, including nut hulls. - Approximately 50% more usable material per pound than mica. - No adverse effect on LC50 values. - Temperature stable to 500 F in all fluids - Does not weaken or deteriorate during use. - Chemically inert. - Compatible with all water, oil, and synthetic based drilling fluids. |  |
| --- | --- |